WORKSHOP INSTRUCTION MANUAL

Electric Propulsion System Digital Twin

Fault Detection using Machine Learning

Workshop Overview Part 1: Build propulsion system model in MATLAB Simscape Part 2: Generate healthy and faulty data using MATLAB scripts Part 3: Train ML fault predictor in Python (Google Colab)

# Table of Contents

Table of Contents	1

Part 1: Building the Simscape Model	1

1.1 System Specifications	1

1.2 Create New Simulink Model	1

1.3 Add Required Blocks	1

1.4 Configure Battery Block	1

1.5 Configure Motor & Drive Block	1

1.6 Create Propeller MATLAB Function	1

1.7 Wire the Model	1

1.8 Add Data Logging	1

1.9 Configure Solver & Run	1

Part 2: Generating Fault Data	1

2.1 Fault Definitions	1

2.2 Run Data Generation Script	1

What the Script Does	1

Output Files	1

Part 3: Building the ML Fault Predictor	1

3.1 Open Google Colab	1

3.2 Upload Your Dataset	1

3.3 Run the Notebook Cells	1

3.4 Understanding the Results	1

3.5 Interactive Prediction Demo	1

Workshop Summary	1

What You Accomplished	1

Key Takeaways	1

Next Steps	1

# Part 1: Building the Simscape Model

Build an electric propulsion digital twin with Battery → Motor → Propeller.

## 1.1 System Specifications

| Component | Parameter | Value |
| --- | --- | --- |
| Battery | Nominal Voltage | 201.6 V |
| Battery | Capacity | 100 Ah |
| Battery | Internal Resistance | 0.05 Ω |
| Motor | Max Power | 20,000 W |
| Motor | Efficiency | 88% |
| Motor | Rated Speed | 2500 RPM |
| Propeller | Diameter | 0.9 m |
| Propeller | Thrust Coeff (Kt) | 0.10 |
| Propeller | Torque Coeff (Kq) | 0.060 |

## 1.2 Create New Simulink Model

Step 1: Open MATLAB R2025b and type in Command Window:

>> simulink

Step 2: Click Blank Model to create new model

Step 3: Save as ElectricPropSys_digitalTwin.slx

## 1.3 Add Required Blocks

From Library Browser (Ctrl+Shift+L), add these blocks:

| Block | Library Path |
| --- | --- |
| Battery | Simscape → Electrical → Sources |
| Motor & Drive | Simscape → Electrical → Electromechanical |
| Current Sensor | Simscape → Electrical → Sensors |
| Voltage Sensor | Simscape → Electrical → Sensors |
| Ideal Rotational Motion Sensor | Simscape → Driveline → Sensors |
| Ideal Torque Source | Simscape → Driveline → Sources |
| Inertia | Simscape → Driveline → Inertias & Loads |
| Solver Configuration | Simscape → Utilities |
| Electrical Reference | Simscape → Electrical → Connectors |
| Mechanical Rotational Reference | Simscape → Utilities |
| PS-Simulink Converter | Simscape → Utilities |
| Simulink-PS Converter | Simscape → Utilities |
| MATLAB Function | Simulink → User-Defined Functions |
| Constant (×3) | Simulink → Sources |
| To Workspace (×5) | Simulink → Sinks |

## 1.4 Configure Battery Block

Double-click Battery block and set:

| Parameter | Value |
| --- | --- |
| Nominal voltage, Vnom | 201.6 V |
| Maximum capacity, AH | 100 Ah |
| Internal resistance, R1 | 0.05 Ohm |
| Voltage V1 (at AH1) | 180 V |
| Capacity AH1 | 87 Ah |

## 1.5 Configure Motor & Drive Block

Double-click Motor & Drive block and set:

| Parameter | Value |
| --- | --- |
| Maximum torque | 100 N*m |
| Maximum power | 20000 W |
| Efficiency | 88 % |
| Torque time constant | 0.015 s |

## 1.6 Create Propeller MATLAB Function

Double-click MATLAB Function block and enter:

function [Torque, Thrust] = propeller(omega, Kt_input) Kt = Kt_input; % Thrust coefficient Kq = 0.060; % Torque coefficient rho = 1.225; % Air density (kg/m³) D = 0.9; % Diameter (m) n = abs(omega) / (2 * pi); % rev/s Torque = -Kq * rho * n^2 * D^5; Thrust = Kt * rho * n^2 * D^4; end

💡 TIP: The negative torque indicates the propeller load opposes motor rotation.

## 1.7 Wire the Model

Electrical Connections:

• Battery (+) → Current Sensor → Motor & Drive (+)

• Battery (-) → Motor & Drive (-) → Electrical Reference

• Voltage Sensor across Battery terminals

Mechanical Connections:

• Motor (R) → Motion Sensor (R) → Inertia → Torque Source (R)

• Motor (C) → Mechanical Rotational Reference

Signal Connections:

• Motion Sensor (W) → PS-Simulink → MATLAB Function (omega)

• Constant (Kt_prop=0.10) → MATLAB Function (Kt_input)

• MATLAB Function (Torque) → Simulink-PS → Torque Source (S)

• Constant (76) → Simulink-PS → Motor & Drive (T)

## 1.8 Add Data Logging

Add To Workspace blocks to log signals (set Save format = Timeseries):

| Signal | Variable Name | Source |
| --- | --- | --- |
| Voltage | V_battery | Voltage Sensor |
| Current | I_battery | Current Sensor |
| Speed | omega | Motion Sensor (W) |
| Torque | torque | MATLAB Function |
| Thrust | thrust | MATLAB Function |

## 1.9 Configure Solver & Run

Go to Simulation → Model Configuration Parameters:

| Setting | Value |
| --- | --- |
| Stop time | 10 |
| Solver | ode23t (mod. stiff/Trapezoidal) |
| Max step size | 0.001 |

Set workspace variables and run:

>> Kt_prop = 0.10; >> sim('ElectricPropSys_digitalTwin');

Expected: ~2500 RPM, ~139 N thrust at steady state

📝 NOTE: Save your model before continuing to Part 2!

# Part 2: Generating Fault Data

Generate training data by injecting faults and running multiple simulations.

## 2.1 Fault Definitions

| Label | Fault | Parameter Change | Physical Cause |
| --- | --- | --- | --- |
| 0 | Healthy | Nominal values | Normal operation |
| 1 | Battery | R: 0.05 → 0.15 Ω | Aged cells, corrosion |
| 2 | Motor | Efficiency: 88 → 70% | Worn bearings |
| 3 | Propeller | Kt: 0.10 → 0.06 | Blade damage |

## 2.2 Run Data Generation Script

Step 1: Open simscape_fault_dataset_generator.m

Step 2: Verify model name matches your file

Step 3: Run the script (F5)

Step 4: Wait for simulations to complete (~5-10 minutes)

### What the Script Does

1. Loads your Simscape model

2. For each fault condition (0-3):

• Sets fault parameters in model blocks

• Runs 50 simulations with varying throttle

• Logs all sensor data

3. Saves dataset to CSV and MAT files

### Output Files

| File | Description |
| --- | --- |
| simscape_fault_dataset.csv | Training data for Python ML |
| simscape_fault_dataset.mat | MATLAB format with metadata |

📝 NOTE: Keep the CSV file - you will upload it to Google Colab in Part 3!

# Part 3: Building the ML Fault Predictor

Train a machine learning model in Python to classify faults from sensor data.

## 3.1 Open Google Colab

Step 1: Go to https://colab.research.google.com

Step 2: Sign in with your Google account

Step 3: Click File → Upload notebook and select the workshop notebook

## 3.2 Upload Your Dataset

Run the first code cell to upload your CSV file:

from google.colab import files uploaded = files.upload() # Select simscape_fault_dataset.csv

## 3.3 Run the Notebook Cells

Execute cells in order using Shift+Enter:

| Cell | Purpose | Expected Output |
| --- | --- | --- |
| 1-2 | Setup & Upload | Libraries imported, file uploaded |
| 3-4 | Load & Explore | Dataset shape, class distribution |
| 5-6 | Visualize | Histograms, scatter plots |
| 7 | Feature Engineering | 5 new features created |
| 8 | Train-Test Split | 80% train, 20% test |
| 9 | Train Models | 6 models trained |
| 10 | Compare Models | Accuracy comparison table |
| 11 | Best Model | Confusion matrix, metrics |
| 12 | Feature Importance | Top features ranked |
| 13 | Save Model | Files downloaded |
| 14 | Live Demo | Interactive fault prediction |

## 3.4 Understanding the Results

Model Performance:

• Random Forest and SVM typically achieve 90-100% accuracy

• Neural Network also performs well on this dataset

Key Features for Fault Detection:

| Fault Type | Key Indicator | Observable Change |
| --- | --- | --- |
| Battery | Voltage Drop | Lower V at same current |
| Motor | Efficiency | High P_elec, low P_mech |
| Propeller | Thrust/Power Ratio | Low thrust at same RPM |

## 3.5 Interactive Prediction Demo

The last cell has interactive sliders. Try these scenarios:

• Healthy: Voltage ~196V, Efficiency ~86%

• Battery Fault: Lower voltage to ~183V

• Motor Fault: Lower efficiency to ~70%

• Propeller Fault: Lower thrust to ~75N

💡 TIP: Watch how the model predicts different faults as you adjust the sliders!

# Workshop Summary

## What You Accomplished

✓ Built an electric propulsion digital twin in MATLAB Simscape

✓ Learned to inject faults (battery, motor, propeller)

✓ Generated labeled training data from simulations

✓ Trained multiple ML models for fault classification

✓ Evaluated model performance with confusion matrices

✓ Built an interactive fault prediction demo

## Key Takeaways

• Digital twins enable safe fault simulation without damaging real systems

• Feature engineering (derived features) improves ML model accuracy

• Different faults have distinct signatures in sensor data

• Ensemble methods (Random Forest) often outperform single models

## Next Steps

• Collect more training data for improved accuracy

• Add fault severity levels (mild, moderate, severe)

• Deploy model in real-time monitoring system

• Explore other ML architectures (LSTM for time-series)

• Integrate with hardware-in-the-loop testing